# CIVE 202 Project #2 Report

Group B – Fatima, Macy, Lucas

2/24/2026

# Table of Contents

Table of Contents	1

Table of Figures	2

Introduction	2

Methods	3

Results	5

References	8

# Table of Figures

Table 1 Input Variables	4

Table 2 Output Variables	5

Table 3 Scenario 01 Summary Table	6

Table 4 Scenario 02 Summary Table	7

Table 5 Scenario 03 Summary Table	7

Table 6 Scenario 04 Summary Table	8

# Introduction

The Nebraska Department of Transportation (NDOT) currently uses a Microsoft Excel sheet to perform calculations for concrete mix design. NDOT wishes to migrate this calculation to work to a Python-based system, which is the goal of this project. The Python program will prompt users to enter material properties and design parameters in a step-by-step manner, and then it will produce an automatically generated weight chart for one cubic yard of concrete.

By transferring its concrete mix design calculations from Excel to Python, NDOT intends to improve the reliability of its mix design process. Migrating to Python will ensure that the logic used in calculations is fixed, transparent, and repeatable. This will make the calculation logic more suitable for use in a future web-based tool.

# Methods

Converting the mix design calculation logic from Excel to Python was relatively simple. The logic and necessary variables are all defined in the Excel sheet currently used for calculations, and the logic is written in a way that is already compatible with Python. The variables used in the Python code are also the same as the variables used in the Excel sheet.

For the variables that are inputs (Table 1), the user is prompted to input each variable, one at a time. The prompts include which units the input should be in.

- Table 1 Input Variables

| Letter | Description | Units |
| --- | --- | --- |
| A | Weight of Cement | lb/yd^3 |
| B | Weight of Fly Ash | lb/yd^3 |
| C | Weight of Silica Fume | lb/yd^3 |
| D | Weight of other SCM | lb/yd^3 |
| E | Target Water/Cement Ratio | No units |
| F | Target % Air Content | No Units |
| G | Target %Fine Aggregate | No units |
| H | Target % Coarse Aggregate | No units |
| I | Target % Other Aggregate | No units |
| J | Specific Gravity of Cement | No units |
| K | Specific Gravity of Fly Ash | No units |
| L | Specific Gravity of Silica Fume | No units |
| M | Specific Gravity of other SCM | No units |
| N | Specific Gravity of Fine Aggregate | No units |
| O | Specific Gravity of Coarse Aggregate | No units |
| P | Specific Gravity of Other Aggregate | No units |

For the variables that are outputs (Table 2), a function has been written in Python to calculate the value of each variable using the inputs provided by the user and the logic from the Excel sheet. Once the user has input the value of each input variable, the output variables will be automatically produced and presented with their respective units.

- Table 2 Output Variables

| Letter | Description | Formula | Units |
| --- | --- | --- | --- |
| Q | Weight of Water | (A+B+C+D)*E | lb |
| R | Volume of Cement | A/(J*62.4) | ft^3 |
| S | Volume of Fly Ash | B/(K*62.4) | ft^3 |
| T | Volume of Silica Fume | C/(L*62.4) | ft^3 |
| U | Volume of other SCM | D/(M*62.4) | ft^3 |
| V | Volume of Air | (F/100)*27 | ft^3 |
| W | Volume of Water | Q/62.4 | ft^3 |
| X | Volume of Total Aggregate | 27-R-S-T-U-V-W | ft^3 |
| Y | Weight of Fine Aggregate | 62.4*(G/100)*N*X | lb |
| Z | Weight of Coarse Aggregate | 62.4*(H/100)*O*X | lb |
| AA | Weight of Other Aggregate | 62.4*(I/100)*P*X | lb |

# Results

The four mix designs our team chose were Standard Pavement (class47B), Bridge Deck (class 47 BR), Low Carbon Concrete (LCCI), and High Early Strength (PR3). The Standard Pavement mix represents a typical highway paving design with moderate cement content (Nebraska Department of Transportation), a balanced water-cement ratio, and an air entrainment for freeze-thaw durability, which results in a stable and economical aggregate distribution. In comparison, the Bridge Deck mix incorporates fly ash and slightly higher air content to enhance durability, reduce permeability, and mitigate alkali-silica reaction. This then slightly lowers cement demand while adjusting total water aggregate volumes. The Low Carbon Concrete mix reduces overall cement content and increases cementitious materials to minimize carbon emissions (Morcous, G., & Hu, J.). High Early Strength mix uses a higher cement content and lower water-cement ratio to get rapid strength gain, which increases cement volume.

Table 3 Scenario 01 Summary Table

---------------------------------------------

NDOT Concrete Mix Design – Weight Summary

(1 Cubic Yard of Concrete)

---------------------------------------------

Project Number:       1

Class of Concrete:    47B

---------------------------------------------

Cement (A):               564.0 lb

Fly Ash (B):                0.0 lb

Silica Fume (C):            0.0 lb

Other SCM (D):              0.0 lb

---------------------------------------------

Fine Aggregate (Y):      2095.2 lb

Coarse Aggregate (Z):     911.7 lb

Other Aggregate (AA):       0.0 lb

---------------------------------------------

Water (Q):                253.8 lb

---------------------------------------------

Table 4 Scenario 02 Summary Table

---------------------------------------------

NDOT Concrete Mix Design – Weight Summary

(1 Cubic Yard of Concrete)

---------------------------------------------

Project Number:       2

Class of Concrete:    47BR

---------------------------------------------

Cement (A):               510.0 lb

Fly Ash (B):              145.0 lb

Silica Fume (C):            0.0 lb

Other SCM (D):              0.0 lb

---------------------------------------------

Fine Aggregate (Y):      1259.9 lb

Coarse Aggregate (Z):    1421.1 lb

Other Aggregate (AA):     140.5 lb

---------------------------------------------

Water (Q):                275.1 lb

---------------------------------------------

Table 5 Scenario 03 Summary Table

---------------------------------------------

NDOT Concrete Mix Design – Weight Summary

(1 Cubic Yard of Concrete)

---------------------------------------------

Project Number:       3

Class of Concrete:    LCCI

---------------------------------------------

Cement (A):               470.0 lb

Fly Ash (B):              120.0 lb

Silica Fume (C):           25.0 lb

Other SCM (D):              0.0 lb

---------------------------------------------

Fine Aggregate (Y):      1479.3 lb

Coarse Aggregate (Z):    1490.6 lb

Other Aggregate (AA):       0.0 lb

---------------------------------------------

Water (Q):                246.0 lb

---------------------------------------------

Table 6 Scenario 04 Summary Table

---------------------------------------------
 NDOT Concrete Mix Design – Weight Summary
         (1 Cubic Yard of Concrete)
---------------------------------------------
Project Number:       4
Class of Concrete:    PR3
---------------------------------------------
Cement (A):               750.0 lb
Fly Ash (B):                0.0 lb
Silica Fume (C):            0.0 lb
Other SCM (D):              0.0 lb
---------------------------------------------
Fine Aggregate (Y):      1111.5 lb
Coarse Aggregate (Z):    1705.5 lb
Other Aggregate (AA):       0.0 lb
---------------------------------------------
Water (Q):                285.0 lb
---------------------------------------------
End of Mix Design Summary

# References

Morcous, G., & Hu, J. (2015). Optimized aggregate gradations for Portland cement concrete mix designs evaluation (Final Report No. P336). Nebraska Department of Transportation. https://dot.nebraska.gov/media/3isfdv45/final-report-p336.pdf

Morcous, G., & Hu, J. (2019). Evaluation of reducing cement content in NDOR class R combined aggregate gradations (Final Report No. M069). Nebraska Department of Transportation. https://dot.nebraska.gov/media/qqhnyrgi/m069-reducing-cement-project-report.pdf

Nebraska Department of Transportation. (2017). Standard specifications for highway construction. https://dot.nebraska.gov/media/g4qp4y0d/2017-specbook.pdf

Nebraska Department of Transportation. (n.d.). Mix design submittal spreadsheet. https://dot.nebraska.gov/media/jp3paote/mix-design-submittal.xlsx